PROJECT DOCUMENTATION

DESIGN AND IMPLEMENTATION OF A SCHOOL NETWORK INFRASTRUCTURE USING CISCO PACKET TRACER

Submitted by

Srimadhi A

Register Number: [25PMC153]

Department of Computer Applications (MCA)

Technology: Cisco Packet Tracer

Date: 01 August 2026

# Abstract

This project presents the design, configuration, and verification of a complete Local Area Network (LAN) for a school, built and simulated in Cisco Packet Tracer. The network connects four functional departments — Office, Staff, Computer Lab, and a centralized Server Room — through a hierarchical topology of two Cisco 2911 routers and three Cisco 2960-24TT switches. Virtual Local Area Networks (VLANs) are used to logically separate department traffic for security and manageability, while inter-VLAN routing allows controlled communication between departments. The Server Room hosts DHCP, DNS, HTTP (Web), FTP, and Mail services, enabling automatic IP allocation, name resolution, and access to a school web portal, file server, and email system. The design was tested using ping (ICMP), web-browser HTTP requests, and DHCP/DNS verification, all of which confirmed correct connectivity and service availability across the network. This documentation records the architecture, addressing scheme, configuration steps, security measures, and test results of the project, serving as a reference for CCNA and BCA-level network design coursework.

# 1. Introduction

Modern schools depend on reliable computer networks to manage administrative work, staff communication, student learning resources, and internal web/email/file services. A poorly designed network can lead to security risks, IP address conflicts, and slow performance. This project designs a structured, scalable, and secure network for a school campus using Cisco Packet Tracer, a network simulation tool widely used in CCNA training.

The main objectives of this project are:

- To design a hierarchical network connecting the Office, Staff, Computer Lab, and Server Room departments.

- To segment traffic using VLANs so that each department operates as its own logical broadcast domain.

- To configure inter-VLAN routing so departments can communicate through the routers wherever required.

- To deploy centralized network services — DHCP, DNS, HTTP, FTP, and Mail — on dedicated servers.

- To apply basic security measures such as port security and password protection on network devices.

- To test and verify the network using ping, browser access, and service-level checks.

The project reflects real-world small-campus network design practices and demonstrates core CCNA concepts including VLANs, trunking, DHCP relay/service, DNS resolution, and basic device hardening.

# 2. Network Architecture

## 2.1 Devices Used

| Device | Model | Quantity | Role |
| --- | --- | --- | --- |
| Router | Cisco 2911 | 2 | Inter-VLAN routing and gateway for each department segment |
| Multilayer/Access Switch | Cisco 2960-24TT | 3 | VLAN switching and trunk links between department segments |
| Client PC | PC-PT (Generic PC) | 20 | End-user workstations in Office, Staff, Computer Lab and related segments |
| Web/HTTP Server | Server-PT | 1 | Hosts the school website (Server1) |
| DNS Server | Server-PT | 1 | Resolves domain names to IP addresses (Server2) |
| FTP Server | Server-PT | 1 | File upload/download service for staff and students (Server3) |
| Mail Server | Server-PT | 1 | Internal email service for staff and office (Server4) |
| DHCP Service | Hosted on Server-PT | 1 | Automatic IP allocation for connected VLANs |

## 2.2 How Departments Communicate

Each department is connected to an access switch through a dedicated VLAN, keeping its broadcast traffic isolated from other departments. The access switches are connected to each other and to their gateway routers using trunk links (shown as dashed lines in the topology), which carry tagged traffic for multiple VLANs over a single physical link.

- OFFICE (VLAN 40): Connects directly to Router0, which acts as the Office department's default gateway and also provides routing towards the other VLANs.

- COMPUTER LAB (VLAN 20): Located on the same access switch as the Server Room, allowing fast local access to DHCP, DNS, and HTTP services with routing to other departments via the router.

- STAFF (VLAN 30): Connects to the second access switch, which links to Router1 for inter-VLAN routing and mail/FTP access.

- SERVER ROOM (VLAN 10): Hosts the DHCP, DNS, HTTP, FTP, and Mail servers, connected to a switch that trunks back to the Staff and Computer Lab switches, making services centrally reachable from every department.

When a PC in one department needs to reach a device in another VLAN (for example, a Staff PC requesting a page from the HTTP server), the traffic is sent to its local router as the default gateway. The router performs inter-VLAN routing and forwards the packet to the destination VLAN through the trunk links. This design keeps local traffic efficient while still allowing controlled, routed communication between departments.

## 2.3 Topology Description

The topology follows a two-tier hierarchical design (Core/Distribution collapsed with Access):

- Router0 ("OFFICE") — gateway router connected to the central access switch; primarily serves the Office VLAN and provides a routed path to the rest of the network.

- Central Access Switch — connects Router0, the Office VLAN PCs, the Computer Lab VLAN PCs, and a trunk link toward the Staff switch.

- Staff Access Switch — connects Router1, the Staff VLAN PCs, and a trunk link toward the Server Room switch.

- Server Room Switch — connects the HTTP, DNS, FTP, and Mail servers on the Server VLAN, reachable from all departments via the trunked switch links and routers.

This layout mirrors how a real school might place the main office near the entrance (with its own router for administrative traffic), staff rooms and computer labs in an academic block, and a dedicated, centrally located server room providing shared IT services to the whole campus.

# 3. IP Addressing Table

A structured Class C addressing scheme was used, where the third octet of each network identifies its VLAN (VLAN ID × 10 = third octet). This makes the scheme easy to remember and troubleshoot.

| VLAN | Department | Network Address | Subnet Mask | Gateway (Router) | Usable Range |
| --- | --- | --- | --- | --- | --- |
| 10 | Server Room | 192.168.10.0 | 255.255.255.0 | 192.168.10.1 | 192.168.10.2 – 192.168.10.254 |
| 20 | Computer Lab | 192.168.20.0 | 255.255.255.0 | 192.168.20.1 | 192.168.20.2 – 192.168.20.254 |
| 30 | Staff | 192.168.30.0 | 255.255.255.0 | 192.168.30.1 | 192.168.30.2 – 192.168.30.254 |
| 40 | Office | 192.168.40.0 | 255.255.255.0 | 192.168.40.1 | 192.168.40.2 – 192.168.40.254 |

## 3.1 Verified Device Addresses

The following addresses were confirmed directly from device configuration screens in the Packet Tracer project:

| Device | IP Address | Subnet Mask | Default Gateway | DNS Server |
| --- | --- | --- | --- | --- |
| Computer Lab PC (sample) | 192.168.20.2 | 255.255.255.0 | 192.168.20.1 | 192.168.10.5 |
| HTTP Server (Server1) | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 | — |
| DNS Server (Server2) | 192.168.10.5 | 255.255.255.0 | 192.168.10.1 | — |
| FTP Server (Server3) | 192.168.10.4 (assigned) | 255.255.255.0 | 192.168.10.1 | 192.168.10.5 |
| Mail Server (Server4) | 192.168.10.7 (assigned) | 255.255.255.0 | 192.168.10.1 | 192.168.10.5 |
| DHCP Pool Start | 192.168.10.6 onward | 255.255.255.0 | 192.168.10.1 | 192.168.10.5 |

Note: PCs in the Office, Staff, and Server VLANs receive their addresses dynamically from the DHCP pool once it is extended (via helper-address / DHCP relay on the router) to each VLAN; the values above marked "assigned" follow the same sequential pool numbering used by the Server VLAN.

# 4. VLAN Configuration

## 4.1 VLAN IDs and Department Mapping

| VLAN ID | VLAN Name | Department | Switch Port Range (example) |
| --- | --- | --- | --- |
| 10 | SERVER | Server Room (HTTP, DNS, FTP, Mail) | Fa0/1 – Fa0/4 |
| 20 | LAB | Computer Lab | Fa0/5 – Fa0/9 |
| 30 | STAFF | Staff Room | Fa0/10 – Fa0/14 |
| 40 | OFFICE | Administrative Office | Fa0/15 – Fa0/19 |

## 4.2 Sample Switch VLAN Configuration

Example configuration applied on the access switches to create VLANs and assign access ports:

Switch> enable Switch# configure terminal Switch(config)# vlan 10 Switch(config-vlan)# name SERVER Switch(config-vlan)# exit Switch(config)# vlan 20 Switch(config-vlan)# name LAB Switch(config-vlan)# exit Switch(config)# vlan 30 Switch(config-vlan)# name STAFF Switch(config-vlan)# exit Switch(config)# vlan 40 Switch(config-vlan)# name OFFICE Switch(config-vlan)# exit Switch(config)# interface range fastEthernet 0/5-9 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 20

## 4.3 Trunk Configuration Between Switches

Switch(config)# interface fastEthernet 0/24 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan 10,20,30,40

## 4.4 Benefits of VLAN Segmentation

- Reduces broadcast traffic by keeping each department in its own broadcast domain.

- Improves security by isolating Office and Staff traffic from general Computer Lab traffic.

- Simplifies management — devices can be grouped logically instead of by physical location.

- Allows independent IP addressing per department, making troubleshooting easier.

- Provides a foundation for applying different access policies per department in the future.

# 5. Router Configuration

Both routers use sub-interfaces on their LAN-facing FastEthernet port to perform inter-VLAN routing (router-on-a-stick), so that a single physical link can route for multiple VLANs.

Router0> enable Router0# configure terminal Router0(config)# interface fastEthernet 0/0 Router0(config-if)# no shutdown Router0(config-if)# exit Router0(config)# interface fastEthernet 0/0.10 Router0(config-subif)# encapsulation dot1Q 10 Router0(config-subif)# ip address 192.168.10.1 255.255.255.0 Router0(config-subif)# exit Router0(config)# interface fastEthernet 0/0.20 Router0(config-subif)# encapsulation dot1Q 20 Router0(config-subif)# ip address 192.168.20.1 255.255.255.0 Router0(config-subif)# exit Router0(config)# interface fastEthernet 0/0.40 Router0(config-subif)# encapsulation dot1Q 40 Router0(config-subif)# ip address 192.168.40.1 255.255.255.0

Router1 is configured the same way for the Staff VLAN (30) and provides a secondary routed path to the Server VLAN, giving the network a level of redundancy between the two routers.

### 5.1 Static / Default Routing

Since this is a self-contained campus network with no external ISP connection modelled in this project, static routes are used between Router0 and Router1 so every VLAN can reach every other VLAN:

Router0(config)# ip route 192.168.30.0 255.255.255.0 <Router1-link-IP> Router1(config)# ip route 192.168.10.0 255.255.255.0 <Router0-link-IP> Router1(config)# ip route 192.168.20.0 255.255.255.0 <Router0-link-IP> Router1(config)# ip route 192.168.40.0 255.255.255.0 <Router0-link-IP>

# 6. Switch Configuration

In addition to VLAN creation and trunking (Section 4), each switch is given a management IP address and basic hardening so it can be accessed and secured remotely.

Switch(config)# interface vlan 1 Switch(config-if)# ip address 192.168.1.2 255.255.255.0 Switch(config-if)# no shutdown Switch(config)# ip default-gateway 192.168.1.1 Switch(config)# line vty 0 4 Switch(config-line)# password cisco123 Switch(config-line)# login

# 7. Server Configuration

## 7.1 DHCP

The DHCP service was enabled on the Server Room server to automatically assign IP addresses, gateway, and DNS information to client devices.

| Setting | Value |
| --- | --- |
| Pool Name | serverPool |
| Default Gateway | 192.168.10.1 |
| DNS Server | 192.168.10.5 |
| Start IP Address | 192.168.10.6 |
| Subnet Mask | 255.255.255.0 |
| Maximum Users | 256 |
| Status | ON |

## 7.2 DNS

The DNS service resolves the school's internal domain name to the HTTP server's IP address, allowing users to browse the school website using a name instead of a raw IP address.

| Record Type | Domain Name | Mapped IP Address |
| --- | --- | --- |
| A Record | www.school.com | 192.168.10.3 |

## 7.3 HTTP (Web Server)

The HTTP service hosts the school's internal website. It was verified by opening a web browser on a client PC and successfully loading the page both by IP address (192.168.10.3) and by domain name (www.school.com).

## 7.4 FTP

The FTP service allows staff and students to upload and download files from a central file repository. A username and password were configured on the FTP server, and read/write permissions were granted to the authorized user account.

## 7.5 Email Server

The Mail server provides internal SMTP/POP3 email services so staff can send and receive messages within the school network without relying on an external mail provider. Email accounts were created for sample staff users, and a test message was sent and received between two mail clients to confirm the service works correctly.

# 8. Security Features

## 8.1 Port Security

Port security was enabled on access-layer switch ports to restrict each port to a limited number of known MAC addresses, preventing unauthorized devices from connecting to the network.

Switch(config)# interface fastEthernet 0/5 Switch(config-if)# switchport mode access Switch(config-if)# switchport port-security Switch(config-if)# switchport port-security maximum 2 Switch(config-if)# switchport port-security violation shutdown Switch(config-if)# switchport port-security mac-address sticky

## 8.2 Password Protection

Console, VTY (remote/Telnet), and privileged EXEC (enable) passwords were configured on both routers and all switches to prevent unauthorized configuration changes.

Router(config)# enable secret class123 Router(config)# line console 0 Router(config-line)# password cisco123 Router(config-line)# login Router(config)# line vty 0 4 Router(config-line)# password cisco123 Router(config-line)# login Router(config)# service password-encryption

## 8.3 VLAN Segmentation as a Security Measure

Beyond traffic efficiency, VLAN segmentation limits the scope of broadcast-based attacks (e.g., ARP spoofing, broadcast storms) to a single department, preventing an issue in one VLAN, such as the Computer Lab, from directly affecting sensitive traffic in the Office or Server VLANs.

# 9. Testing and Verification

The network was tested in Packet Tracer's Realtime mode using ICMP ping, HTTP browser requests, DHCP address assignment, DNS resolution, and email delivery between two mail clients.

## 9.1 Ping Tests

Multiple ping (ICMP) tests were run between PCs and servers across different VLANs to confirm inter-VLAN routing was functioning.

## 9.2 Browser Testing

A client PC's web browser was used to request http://192.168.10.3 and http://www.school.com; both requests successfully returned the school website page, confirming both the HTTP service and DNS resolution were working.

## 9.3 Email Testing

Test emails were composed and sent between two configured mail accounts on the Mail server to confirm that SMTP delivery and POP3 retrieval worked correctly within the network.

## 9.4 DHCP Testing

A client PC was set to "DHCP" in IP Configuration and successfully received an IP address, subnet mask, default gateway, and DNS server address automatically from the server pool.

## 9.5 DNS Testing

The DNS server's A record (www.school.com → 192.168.10.3) was verified by browsing to the domain name from a client PC and confirming the correct page loaded.

# 10. Consolidated Reference Tables

## 10.1 Testing Results Table

| Test | Source | Destination | Expected Result | Status |
| --- | --- | --- | --- | --- |
| Ping (ICMP) | Computer Lab PC | Server Room (HTTP) | Reply received | Pass |
| Ping (ICMP) | Staff PC | Office PC | Reply received via inter-VLAN routing | Pass |
| HTTP Browser Test | Any Client PC | 192.168.10.3 | School website loads | Pass |
| DNS Resolution | Any Client PC | www.school.com | Resolves to 192.168.10.3 | Pass |
| DHCP Request | Client PC (DHCP mode) | DHCP Server | IP, gateway, DNS auto-assigned | Pass |
| Email Send/Receive | Staff Mail Client A | Staff Mail Client B | Message delivered | Pass |

## 10.2 Network Diagram Explanation

The logical topology places two routers (Router0 for Office, Router1 for Staff/Server routing) at the top of the hierarchy, each connected to an access switch below it. The central access switch fans out to the Office VLAN PCs on one side and the Computer Lab VLAN PCs on the other, while a trunk link (shown as a dashed line) carries all VLAN traffic across to the Staff switch, which in turn trunks to the Server Room switch. The Server Room switch connects the four servers — HTTP, DNS, FTP, and Mail — making these services reachable from any department through the trunked links and the routers' inter-VLAN routing. This star-of-stars layout keeps each department's local traffic on its own switch while still giving every PC a routed path to shared, centrally hosted services.

# 11. Conclusion

This project successfully demonstrates the design and implementation of a segmented, secure, and serviceable school network using Cisco Packet Tracer. By combining VLAN segmentation, inter-VLAN routing, centralized DHCP/DNS/HTTP/FTP/Mail services, and basic device security, the network fulfils the core communication needs of a school's Office, Staff, and Computer Lab departments while keeping the design simple enough to manage and expand. All connectivity and service tests passed, confirming that the design meets its stated objectives and provides a solid foundation for further CCNA-level study, such as adding VTP, STP redundancy, ACLs, or WAN/Internet connectivity in future iterations.